BIOCON BIOLOGICS LIMITED

Standard Operating Procedure

Upstream Cell Culture Operations — Monoclonal Antibody (mAb)

| SOP Number | BC-MFG-UC-047 |
| --- | --- |
| Revision | Rev 3 |
| Effective Date | 15-Jan-2025 |
| Department | Biomanufacturing — Upstream Processing |
| Product | Trastuzumab Biosimilar (BCD-Trast-001) |
| Author | P. Krishnamurthy, Senior Process Scientist |
| Reviewer | S. Iyer, QA Manager |
| Approver | R. Narayanan, Head of Manufacturing |
| Next Review Date | 15-Jan-2026 |

# Pre-Scripted Compliance Gap Summary

NOTE: The following gaps are intentionally present in this SOP for demo purposes. They map to real regulatory clauses and will be detected by the RAG gap engine.

| SOP Clause | Regulation Reference | Gap Description | Severity |
| --- | --- | --- | --- |
| Section 7.3 — Process Change Control | EMA CHMP/437/04 Rev1 §5.2.3 | No comparability protocol defined for post-approval manufacturing changes | CRITICAL |
| Section 6.2 — Acceptance Criteria | EMA BWP/247713 §5.3.2 | Glycosylation profile not listed as a Critical Quality Attribute (CQA) | CRITICAL |

# 1. Purpose and Scope

This SOP defines the operating procedures for upstream cell culture operations in the production of trastuzumab biosimilar (BCD-Trast-001) using Chinese Hamster Ovary (CHO) cell lines at the Biocon Biologics Bengaluru manufacturing facility.

This document applies to all manufacturing personnel involved in inoculum preparation, bioreactor operation, cell culture monitoring, and harvest activities from N-2 seed expansion through N-stage production bioreactor.

# 2. Regulatory References

- ICH Q10 — Pharmaceutical Quality System

- ICH Q11 — Development and Manufacture of Drug Substances (Biologics)

- 21 CFR Part 211 — Current Good Manufacturing Practice

- EMA/CHMP/BWP/247713/2012 — Guideline on similar biological medicinal products containing biotechnology-derived proteins: quality issues

- EMA CHMP/437/04 Rev1 — Guideline on similar biological medicinal products

# 3. Responsibilities

| Process Scientist | Execute cell culture operations per this SOP; record all data in batch records |
| --- | --- |
| Shift Supervisor | Verify critical process parameters at each stage; sign off batch records |
| QA Representative | Review completed batch records; approve deviations |
| Cell Culture Lead | Authorize any in-process adjustments within defined ranges |

# 4. Materials and Equipment

## 4.1 Equipment

- 5L seed bioreactor (Sartorius BIOSTAT B-DCU II)

- 200L N-1 bioreactor (Sartorius BIOSTAT STR 200)

- 2000L N-stage production bioreactor (Sartorius BIOSTAT STR 2000)

- Offline cell counter (Vi-CELL XR, Beckman Coulter)

- pH/DO calibration equipment

- Biosafety cabinet (Class II, Type A2)

## 4.2 Reagents and Media

- CD CHO Medium (Gibco, Cat No. 10743029)

- HyClone CDM4CHO supplement (Cytiva)

- L-Glutamine 200mM solution

- Anti-foam C emulsion (Sigma-Aldrich)

- Working Cell Bank (WCB) vials — BCD-Trast-WCB-2024

# 5. Cell Thaw and Inoculum Preparation

## 5.1 Vial Thaw

- Retrieve WCB vial from liquid nitrogen storage. Record vial ID, storage location, and retrieval time in batch record (Form BC-MFG-FR-021).

- Thaw vial rapidly in 37°C water bath for no more than 2 minutes. Wipe exterior with 70% IPA before transfer to BSC.

- Transfer cell suspension to pre-warmed T-flask containing 30 mL CD CHO medium.

- Incubate at 37°C ± 0.5°C, 5% CO2, 110 rpm in orbital shaker incubator.

- Perform viability assessment after 24 hours using Vi-CELL XR.

Acceptance Criteria — Post-thaw viability:

- Cell viability must be ≥ 85% at 24 hours post-thaw to proceed to seed expansion.

- Cell density must be ≥ 0.3 x 10^6 cells/mL at 24 hours.

- If viability is < 85%, initiate deviation report (Form BC-QA-DEV-001) and notify QA.

## 5.2 Seed Expansion — N-2 and N-1 Stages

- Inoculate N-2 shake flask at 0.3 x 10^6 viable cells/mL in 200 mL CD CHO medium.

- Culture for 3-4 days until viability ≥ 90% and cell density 2.0–4.0 x 10^6 cells/mL.

- Transfer to N-1 bioreactor at inoculation density of 0.5 x 10^6 viable cells/mL.

- N-1 bioreactor operating parameters: pH 7.1 ± 0.1, DO 40% ± 5%, Temperature 37.0°C ± 0.5°C, Agitation 150 rpm ± 10 rpm.

Acceptance Criteria — N-1 Stage End:

- Cell viability must be ≥ 90% at time of inoculation to N-stage bioreactor.

- Viable cell density: 3.0–6.0 x 10^6 cells/mL.

- Glucose concentration: ≥ 3.0 g/L.

# 6. Production Bioreactor Operation

## 6.1 Inoculation and Critical Process Parameters

Inoculate the 2000L production bioreactor at a target density of 0.5 ± 0.1 x 10^6 viable cells/mL. The following CPPs must be maintained throughout the production run:

| Parameter | Setpoint | Operating Range | Action Limit |
| --- | --- | --- | --- |
| Temperature (°C) | 36.5 | 36.0–37.0 | < 35.5 or > 37.5 |
| pH | 7.05 | 6.95–7.15 | < 6.80 or > 7.30 |
| Dissolved Oxygen (%) | 40 | 35–45 | < 25 |
| Agitation (rpm) | 90 | 80–100 | < 70 |
| CO2 (sparge, slpm) | 0.05 | 0.03–0.08 | > 0.15 |

## 6.2 In-Process Monitoring and Acceptance Criteria

The following parameters shall be measured and recorded daily in the batch record:

- Viable cell density (VCD) and cell viability — daily via Vi-CELL XR

- Glucose and lactate — daily via BioProfile FLEX

- pH and DO — continuous online monitoring

- Titer — every 48 hours via Protein A HPLC

[GAP - CRITICAL] Glycosylation profile (G0F, G1F, G2F, afucosylation %) is NOT listed as an in-process CQA in this SOP. EMA/CHMP/BWP/247713/2012 §5.3.2 requires that glycosylation be explicitly monitored and controlled as a CQA for mAb biosimilars. The SOP must include: (a) glycosylation assay frequency, (b) acceptance ranges aligned to reference product, (c) action to be taken if profile drifts. This gap will cause non-conformance during EMA biosimilar comparability assessment.

Harvest Acceptance Criteria:

- Final harvest viability: ≥ 70% (if viability falls below 70%, consult Cell Culture Lead before proceeding)

- Final titer: ≥ 1.5 g/L

- Bioburden: < 1 CFU/10mL

# 7. Process Change Control and Comparability

## 7.1 Minor Process Changes

Minor changes (e.g., media lot changes, minor equipment substitutions within validated range) shall be documented via Change Control Form BC-QA-CC-001 and approved by QA prior to implementation.

## 7.2 Major Process Changes

Major changes (e.g., bioreactor scale change, cell bank generation, media formulation change) require submission of a Process Change Notification (PCN) to Regulatory Affairs and approval prior to implementation.

## 7.3 Post-Approval Manufacturing Changes — Comparability Protocol

[GAP - CRITICAL] This SOP does not define or reference a comparability protocol for post-approval manufacturing changes. EMA CHMP/437/04 Rev1 §5.2.3 explicitly requires that any change to the manufacturing process of a biosimilar after approval must be supported by a formal comparability exercise demonstrating that product quality, safety, and efficacy are not adversely affected. Required elements missing from this SOP: (1) Trigger criteria for initiating a comparability study; (2) Analytical panel to be used (physicochemical, biological activity, PK); (3) Acceptance criteria for comparability; (4) Protocol approval workflow. Absence of this protocol constitutes a critical gap against EMA biosimilar guidelines.

# 8. Deviation Management

Any deviation from CPPs or acceptance criteria must be:

- Documented in the batch record at the time of occurrence.

- Reported to the Shift Supervisor immediately.

- Assigned a deviation number and investigated per SOP BC-QA-DEV-002.

- Reviewed and closed by QA before batch disposition.

# 9. Document Revision History

| Revision | Date | Description of Change | Author |
| --- | --- | --- | --- |
| Rev 1 | 12-Mar-2022 | Initial release | P. Krishnamurthy |
| Rev 2 | 04-Aug-2023 | Updated N-1 acceptance criteria; added anti-foam procedure | S. Iyer |
| Rev 3 | 15-Jan-2025 | Revised CPP table; updated harvest criteria | R. Narayanan |